ap>en

Password: ( mdp = ‘Cisco’)

ap(config)#int bvi 1

ap(config-if)#ip addr 192.168.11.9 255.255.255.0

ap(config-if)#end

Wifi Personnel :

ap(config)#dot11 ssid societe13_perso

ap(config-ssid)#authentication open

ap(config-ssid)#authentication key-management wpa

guest-mode

ap(config-ssid)# wpa-psk ascii 0 Progtr00

int Dot11Radio0

encryption mode ciphers aes-ccm

ssid societe13_perso

no shutdown

Wifi guest :

ap(config)#dot11 ssid societe13_guest

ap(config-ssid)#authentication open

ap(config-ssid)#authentication key-management wpa

ap(config-ssid)#guest-mode

ap(config-ssid)# wpa-psk ascii 0 Progtr00

int Dot11Radio1

encryption mode ciphers aes-ccm

ssid societe13_guest

no shutdown

ap(config-if)#do sh run

Building configuration...

Current configuration : 1940 bytes

!

! Last configuration change at 08:08:11 UTC Fri Mar 1 2002

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname ap

!

!

logging rate-limit console 9

enable secret 5 $1$waVJ$JX9laN73XwbKBNSEZGeVE/

!

no aaa new-model

no ip cef

!

!

!

!

dot11 syslog

!

dot11 ssid societe13_guest

authentication open

authentication key-management wpa

guest-mode

wpa-psk ascii 7 023616540C121D711C

!

dot11 ssid societe13_perso

authentication open

authentication key-management wpa

guest-mode

wpa-psk ascii 7 1335051D0C18167A7B

!

!

dot11 guest

!

!

!

username Cisco password 7 1531021F0725

!

!

bridge irb

!

!

!

interface Dot11Radio0

no ip address

!

encryption mode ciphers aes-ccm

!

ssid societe13_perso

!

antenna gain 0

channel 2437

station-role root

bridge-group 1

bridge-group 1 subscriber-loop-control

bridge-group 1 spanning-disabled

bridge-group 1 block-unknown-source

no bridge-group 1 source-learning

no bridge-group 1 unicast-flooding

!

interface Dot11Radio1

no ip address

!

encryption mode ciphers aes-ccm

!

ssid societe13_guest

!

antenna gain 0

peakdetect

no dfs band block

channel dfs

station-role root

bridge-group 1

bridge-group 1 subscriber-loop-control

bridge-group 1 spanning-disabled

bridge-group 1 block-unknown-source

no bridge-group 1 source-learning

no bridge-group 1 unicast-flooding

!

interface GigabitEthernet0

no ip address

duplex auto

speed auto

bridge-group 1

bridge-group 1 spanning-disabled

no bridge-group 1 source-learning

!

interface BVI1

ip address 192.168.10.10 255.255.255.0

ipv6 address dhcp

ipv6 address autoconfig

ipv6 enable

!

ip forward-protocol nd

ip http server

no ip http secure-server

ip http help-path http://www.cisco.com/warp/public/779/smbiz/prodconfig/help/eag

!

!

bridge 1 route ip

!

!

!

line con 0

line vty 0 4

login local

transport input all

!

end

Coté siège :

ap>en

Password: ( mdp = ‘Cisco’)

ap(config)#int bvi 1

ap(config-if)#ip addr 192.168.10.8 255.255.255.0

ap(config-if)#end

Wifi Personnel :

ap(config)#dot11 ssid siege8_perso

ap(config-ssid)#authentication open

ap(config-ssid)#authentication key-management wpa

guest-mode

ap(config-ssid)# wpa-psk ascii 0 Progtr00

int Dot11Radio0

encryption mode ciphers aes-ccm

ssid siege8_perso

no shutdown

Wifi guest :

ap(config)#dot11 ssid siege8_guest

ap(config-ssid)#authentication open

ap(config-ssid)#authentication key-management wpa

ap(config-ssid)#guest-mode

ap(config-ssid)# wpa-psk ascii 0 Progtr00

int Dot11Radio1

encryption mode ciphers aes-ccm

ssid siege8_guest

no shutdown