version 15.8

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname router_sucursale

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

!

!

!

!

!

ip dhcp excluded-address 192.168.11.1

ip dhcp excluded-address 192.168.21.1

ip dhcp excluded-address 192.168.31.1

ip dhcp excluded-address 192.168.41.1

ip dhcp excluded-address 192.168.11.9

ip dhcp excluded-address 192.168.11.10

!

ip dhcp pool vlan11

network 192.168.11.0 255.255.255.0

default-router 192.168.11.1

dns-server 192.168.11.9

!

ip dhcp pool vlan21

network 192.168.21.0 255.255.255.0

default-router 192.168.21.1

dns-server 192.168.11.9

!

ip dhcp pool vlan31

network 192.168.31.0 255.255.255.0

default-router 192.168.31.1

dns-server 192.168.11.9

!

ip dhcp pool vlan41

network 192.168.41.0 255.255.255.0

default-router 192.168.41.1

dns-server 192.168.11.9

!

!

!

ip cef

no ipv6 cef

multilink bundle-name authenticated

!

!

!

license udi pid C927-4P sn FGL2348L2DZ

!

!

vtp version 2

!

redundancy

!

!

controller VDSL 0

!

!

!

!

!

interface Tunnel1

ip address 192.168.90.2 255.255.255.0

ip mtu 1400

ip tcp adjust-mss 1360

tunnel source 10.8.2.1

tunnel destination 10.8.1.1

!

interface ATM0

no ip address

shutdown

no atm ilmi-keepalive

!

interface Ethernet0

no ip address

shutdown

!

interface GigabitEthernet0

no ip address

!

interface GigabitEthernet1

switchport trunk allowed vlan 1-1005

switchport mode trunk

no ip address

!

interface GigabitEthernet2

no ip address

!

interface GigabitEthernet3

no ip address

!

interface GigabitEthernet4

ip address 10.8.2.1 255.0.0.0

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

!

interface Vlan1

no ip address

!

interface Vlan10

ip address 192.168.11.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface Vlan20

ip address 192.168.21.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface Vlan30

ip address 192.168.31.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface Vlan40

ip address 192.168.41.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

ip forward-protocol nd

no ip http server

no ip http secure-server

!

!

ip nat inside source list 1 interface GigabitEthernet4 overload

ip nat inside source list 2 interface GigabitEthernet4 overload

ip nat inside source list 3 interface GigabitEthernet4 overload

ip nat inside source list 4 interface GigabitEthernet4 overload

ip route 0.0.0.0 0.0.0.0 192.168.90.1

!

!

!

tftp-server flash:/firmware/vadsl_module_img.bin

access-list 1 permit 192.168.11.0 0.0.0.255

access-list 2 permit 192.168.21.0 0.0.0.255

access-list 3 permit 192.168.31.0 0.0.0.255

access-list 4 permit 192.168.41.0 0.0.0.255

!

control-plane

!

!

line con 0

line 4

no activation-character

transport preferred none

transport input all

transport output all

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

ntp server 34.202.215.187

ntp server pool.ntp.org

!

end