version 15.6

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname router_siege

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

ethernet lmi ce

!

!

!

!

!

!

!

!

!

!

!

!

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

license udi pid CISCO2911/K9 sn FCZ211840CA

!

!

!

redundancy

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

interface Tunnel1

ip address 192.168.90.1 255.255.255.0

ip mtu 1400

ip tcp adjust-mss 1360

tunnel source 10.8.1.1

tunnel destination 10.8.2.1

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

ip address 192.168.60.2 255.255.255.0

ip nat inside

ip virtual-reassembly in

duplex auto

speed auto

!

interface GigabitEthernet0/1

ip address 192.168.50.2 255.255.255.0

ip nat inside

ip virtual-reassembly in

duplex auto

speed auto

!

interface GigabitEthernet0/2

ip address 10.8.1.1 255.0.0.0

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

shutdown

!

interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

router rip

version 2

network 10.0.0.0

network 192.168.10.0

network 192.168.20.0

network 192.168.30.0

network 192.168.40.0

network 192.168.50.0

network 192.168.60.0

network 192.168.70.0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip nat inside source list 1 interface GigabitEthernet0/2 overload

ip nat inside source list 2 interface GigabitEthernet0/2 overload

ip nat inside source list 3 interface GigabitEthernet0/2 overload

ip nat inside source list 4 interface GigabitEthernet0/2 overload

ip nat inside source list 5 interface GigabitEthernet0/2 overload

ip nat inside source list 6 interface GigabitEthernet0/2 overload

ip nat inside source list 7 interface GigabitEthernet0/2 overload

ip route 0.0.0.0 0.0.0.0 10.0.0.1

!

!

!

access-list 1 permit 192.168.10.0 0.0.0.255

access-list 2 permit 192.168.20.0 0.0.0.255

access-list 3 permit 192.168.30.0 0.0.0.255

access-list 4 permit 192.168.40.0 0.0.0.255

access-list 5 permit 192.168.50.0 0.0.0.255

access-list 6 permit 192.168.60.0 0.0.0.255

access-list 7 permit 192.168.70.0 0.0.0.255

!

control-plane

!

!

!

line con 0

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

!

end